| STUDENT NAME: |  |  |  | SECTION: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| STUDENT ID: |  |  |  | COURSE ID: |  |  |
| SOs # 6 | KPI 6.1 |  | KPI 6.2 | KPI 6.3 | GRADES |  |
| SCORE |  |  |  |  | R |  |
| ASSIGNMENT | LAB#5 (SUPERPOSITION THEOREM) |  |  |  | Q |  |
| RELEVANT PARTS | DEVELOPING |  | CONDUCTING | ANALYSIS AND INTERPRETATION |  |  |

EE 250/251

BASIC ELECTRICAL ENGINEERING

SUMMER 2021

EXPERIMENT # 5

SUPERPOSITION THEOREM

Note: All the students are directed to do necessary theoretical calculation for the experiments and come for the laboratory session.

OBJECTIVE:

The objective of this laboratory session is to apply and verify the superposition theorem to linear circuits.

EQUIPMENTS REQUIRED:

| Sr. no | Name | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Resistors |  |  |
| 2 | DC variable voltage supply |  |  |
| 3 | Voltmeter |  |  |
| 4 | Ammeter |  |  |
| 5 | MULTISIM Simulator |  |  |

THEORY:

Superposition Theorem: The voltage and current responses in a network from two or more sources acting simultaneously can be obtained as the sum of the responses from each source acting alone with other sources deactivated. A deactivated current source is an open circuit. A deactivated voltage source is a short circuit.

CIRCUIT DIAGRAM:

FIGURE 1.

THE SUPERPOSITION THEOREM

A : MEASUREMENT OF CURRENTS AND VOLTAGE IN CIRCUIT WITH ALL SOURCES ACTIVE.

- Connect the circuit as shown in Figure 1 in MULTISIM Simulator.

- Adjust the voltage of the power supplies to the required values.

- Use the ammeter and voltmeter to measure the current and voltage , IX and VX respectively.

(Note: Ammeter must be placed in series with the branch to be measured.)

- Run the simulator and observe the values of Ix and Vx.

- Record the value in Table 1.

- Verify the result theoretically and record the same in the table 1.

ADD SIMULATION SNAPSHOT HERE:

B : APPLYING THE SUPERPOSITION THEOREM

- Use the same circuit of Figure 1.

- To apply superposition theorem, deactivate all sources and keep one source active.

- For the above circuit, the -5 V, voltage source is made ACTIVE while all other sources are deactivated as shown in Figure 2.

Figure 2.

- Run the simulator and measure  IX1 and VX1  . Record the values in Table 2.

- Verify the result theoretically and record the same in the table 2.

ADD SIMULATION SNAPSHOT HERE:

6. Connect the circuit as shown in Figure 3. The 3V voltage source is made ACTIVE while all other sources are           deactivated.

Figure 3.

7. Run   the simulation and measure Ix2  and Vx2 . Record the values in Table 2.

8. Verify the result theoretically and record the same in the table 2.

ADD SIMULATION SNAPSHOT HERE:

9. Connect the circuit as shown in Figure 4. The -1A current source is made ACTIVE while all other sources are           deactivated.

10. Run   the simulation and measure Ix3  and Vx3 . Record the values in Table 2.

11 . Verify the result theoretically and record the same in the table 2.

Figure 4.

ADD SIMULATION SNAPSHOT HERE:

12. Connect the circuit as shown in Figure 5. The 2A current source is made ACTIVE while all other sources are           deactivated.

13. Run   the simulation and measure Ix4  and Vx4 . Record the values in Table 2.

14. Verify the result theoretically and record the same in the table 2.

Figure 5.

ADD SIMULATION SNAPSHOT HERE:

Note:

Pay attention to the direction of the currents. Current in the same directions are added and currents in the opposite directions are subtracted.

OBSERVATION TABLE 1:

| Value | Theoretical value | Simulated value |
| --- | --- | --- |
| Ix(mA) |  |  |
| Vx(V) |  |  |

OBSERVATION TABLE 2:

| -5V ACTIVE |  |  | 3V ACTIVE |  |  | -1A ACTIVE |  |  | 2A ACTIVE |  |  | ALGEBRIC SUM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| THT |  | SIM | THT | SIM |  | THT |  | SIM | THT |  | SIM | THEORITICAL |  | SIM |
| IX1 |  |  | IX2 |  |  | IX3 |  |  | IX4 |  |  | IX= IX1 + IX2 + IX3 + IX4 |  |  |
| VX1 |  |  | VX2 |  |  | VX3 |  |  | VX4 |  |  | VX= VX1 + VX2 + VX3 + VX4 |  |  |

REMARK:

SUPERPOSITION THEOREM:

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………….