Course #: 804465–Automatic Control Lab #4

PID Controller Design

Objective

- Introduction to PID Controller.

- To determine  a mathematical model of  PID controller.

- To obtain transfer function of the Controller

- Use Simulink to Implement PID Controller to get close loop response of the system (Lab # 3).

- Tune PID controller to get acceptable response, i.e.,

- Fast rise time

- Minimum overshoot

- No steady-state error

Introduction to PID Controller

In this lab tutorial we will study a simple but very useful feedback compensator structure, theProportional-Integral-Derivative (PID) controller. It is currently one of the most frequently usedcontrollers in the process industry. We will observe the effect of each of the PID parameters on theclosed-loop dynamics and demonstrate how to use a PID controller to improve the systemperformance.

Ina PID controller the control variable is generated from a term proportional to the error, a termwhichis the integral of the error, and a term which is the derivative of the error.

Proportional: the error is multiplied by a gain Kp. A very high gain may cause instability,and a verylow gain may cause the system to drift away.

Integral: the integral of the error is taken and multiplied by a gain Ki. The gain can be adjustedtodrive the error to zero in the required time. A too high gain may cause oscillations and atoo low gainmay result in a sluggish response.

Derivative: The derivative of the error is multiplied by a gain Kd. Again, if the gain is toohigh thesystem may oscillate and if the gain is too low the response may be sluggish.

A proportional controller (Kp ) will have the effect of reducing the rise time and will reduce but nevereliminate the steady-state error. An integral control (Ki) will have the effect of eliminating the steadystateerror for a constant or step input, but it may make the transient response slower. A derivative control (Kd) will have the effect of increasing the stability of the system, reducing the overshoot, andimproving the transient response.

The effects of each of controller parameters, , , and on a closed-loop system are summarizedin the table below.

| CL RESPONSE | RISE TIME | OVERSHOOT | SETTLING TIME | S-S ERROR |
| --- | --- | --- | --- | --- |
| Kp | Decrease | Increase | Small Change | Decrease |
| Ki | Decrease | Increase | Increase | Eliminate |
| Kd | Small Change | Decrease | Decrease | No Change |

Mathematical Model of  PID Controller

Consider the following unity feedback system:

The input–output relationship of a PID controller can be expressed as:

ut=Kpet+ 1Ti0tetdt+Tddetdt

where u(t) is the output from the controller and e(t)= w(t)− y(t), in which w(t) is the desired set-point(reference input) and y(t) is the plant output. Ti and Td are known as the integral and derivative actiontime, respectively. The above equation can also be written as:

ut=Kpet+Ki0tetdt+ Kddetdt+u0

Where,

Ki=KpTi  , Kd=KpTd

By taking Laplace we can write the transfer function of a continuous-time PID as:

U(s)E(s)=Kp+ KpTiS+KpTdS

=KdS2+KpS+KiS